6035 Flex Seating

FUNCTIONAL  FEATURES

Folding chair that stands out because of its excellent balance

between comfort and minimum use of space. The seat tips up automatically as well as the armrests.

The folding system lies in flipping up the armrests with the automatic return of the seat in a single movement. Thus, at the same time that the seat folds up automatically, the armrests move up to a vertical position so that the seat folded does not surpass the depth of 350mm. The backrest always keeps its fixed position.

The seat consists of a compact single block of could moulded polyurethane foam that fully covers the internal metal structure of the seat. This structure comprises a curved tubular frame, a mesh of flat springs and hinge pins for the seat to tip up. The polyurethane foam block is covered with a easily removable and replaceable zip-on upholstery. The backrest is made in the same manner as the seat. Ergonomics has been the primary aspect for designing both seat and back cushions and the seat enhances maximum comfort.

Between the upholstery and the foam of both the seat and the back, there is a 5-mm thick TS System fire curtain that keeps fire from reaching the foam, thus preventing the emission of toxic gases and flames.

The seat tips up automatically thanks to an extremely quiet double spring system inside the seat shell (tested to 500,000 cycles) that requires no lubrication of any kind.

The armrests also consist of a single upholstered block Inside the armrest there is also an extremely quiet return spring (tested at 500.000 cycles), that requires no lubrication and which is extremely quiet.

Two aluminium sides linked together by a metal bridge support the set of seat, back and armrests. In this bridge there are located the housings for the hinges, which incorporate a blocking mechanism, where the axes of the seat rest.

In conjunction with the F 45 table, this is an ideal solution for long working sessions and conferences.

# TECHNICAL  characteristics

structure:

•	Steel plate and tube, welded with continuous arc.

polyurethane foam:

•	Seat density: 65 Kg. m3

•	Back density: 57 Kg. m3

paint:

•	Electrostatic powder epoxy

•	Coating thickness: 70-80 micra

•	Grid adherence: 100%

upholstery:

•	Pilling: Index 5 BS 5811

aluminium:

•	Material: UNE L-2630

•	Breaking Load: 20 Kg./mm2

fire standards

England:

BS5852. Section 5. Ignition Sources 0, 1 and 5

Germany:

DIN54342 part 1 and 2

Italy:

CSE RF/4/83 class 1. IM

Spain/France:

UNE 23727/NF 92-503

Upholstery: M1/Foam: M4/Plastic: M3

WEIGHT:

30 Kg.

Volume:

0,18 m3 (K.D.)