# FUNCTIONAL FEATURES

| An ergonomic, multiple, bar-mounted seating system specially developed for public and private waiting areas. A row of seating with armrests assembled in sets of 3, 4 or 5 seats. Seat and backrest are two separate parts. This means that the position of the backrest does not depend on that of the seat, so that it provides optimal support to the back at all times. The chrome-plated arms are an integral part of the assembly and clearly mark the boundaries of each individual seat. |  |
| --- | --- |

Seat and backrest are made of matt-finish IF727 material. They can also be supplied fully upholstered in natural leather or fire-resistant fabric.

The metal structures of this series are painted with black powder epoxy as standard, but chrome-plated finish is also available.

This series can also incorporate, as an option, tables in between the seats or at the end of a row. A double shared table is yet another possibility in the case of seats installed back-to-back.

These seating systems are mounted on structures with moveable L-shaped feet incorporating unobtrusive height-regulators that can be used to compensate for any unevenness in the floor.

## TECNICHAL CHARACTERISTICS

STRUCTURE:

Steel plate and tube, welded with continuous arc.

PAINT:

Electrostatic powder expoy

Coating thickness: 70-80 micra

Grid adherence: 100%

UPHOLSTERY:

Pilling: Index 5 BS 5811

Test on fire resistance performance:

Specifications: 	UNE/23727

NF/92507

ALUMINIUM:

Material: UNE  L-2630

Breaking Load: 20 Kg./ mm2

POLYPROYLENE:

Material:IF-727 Copolymer Polypropylene

Resistance to breakage DIN 53455 : 28 N/ mm2

Resistance to impact DIN 53453: without breakage

CHROME PLATING:

Coating thickness: 20-micron minimum