CLIP-ON SPECIFICATION

NOTICE:  Consult your local Simplex representative or contact Simplex at 800.638.5726 before specifying any optional or custom metal system as shape, material, size, perforation and finish limitations can affect feasibility.  The options provided in parentheses are not always compatible.

LIMITATION:  The Clip-On ceiling system is not intended for use in kitchens, food processing, food preparation and cleanroom applications due to potential harsh environments and aggressive sanitation requirements.  We recommend an independent evaluation before specifying the Clip-On product in these areas.

SECTION 09 51 33

ACOUSTICAL METAL PAN CEILINGS

PART 1 GENERAL

  - RELATED DOCUMENTS

- Drawings and general conditions of contract, including general and supplementary conditions and Division 1 specification sections, apply to work of this section

  - SUMMARY

- Section include

  - Acoustical ceiling panels

  - Grid suspension system

  - Wire hangers, fasteners, main tees, cross tees and wall moldings

- Related sections

  - Division 26 – Electrical – for additional or larger size wires, as required to properly support lighting fixtures, shall be furnished and installed by the electrical work installer

  - Division 23 – Heating, Ventilating and Air-Conditioning (HVAC) – for work to be coordinated with ceilings

  - Division 09 20 00 – Plaster and Gypsum Board – for plaster and gypsum board assemblies

- Alternates

  - Prior Approval:  Unless otherwise provided for in the contract documents, proposed product substitutions may be submitted no later than 10 working days prior to the date established for receipt of bids.  Approval of a proposed substitution is contingent upon the Architect’s review of the proposal for acceptability and approved products will be set forth by addenda

    - If substitute products that have not been approved by Addenda are included in a bid, the specified products shall be provided without additional compensation

  - Submittals:  That do not provide adequate information for the product evaluation will not be considered.  The proposed substitution must meet all requirements of this Section, including but not limited to: Single source materials supplier (if specified in Section 1.05); panel design, size, composition, color and finish; suspension system component profiles and sizes; compliance with the referenced standards

  - REFERENCES

- ASTM E 84 “Standard Test Method for Surface Burning Characteristics of Building Materials”

- ASTM A 366 “Standard Specification for Steel, Carbon Cold-Rolled Sheet, Commercial Quality”

- ASTM C 423 “Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method”

- ASTM C 635 “Standard Specification for Metal Suspension Systems for Acoustical Tile and Lay-in Panel Ceilings”

- ASTM C 636 “Recommended Practice for Installation of Metal Ceiling Suspension Systems for Acoustical and Lay-in Panels”

- ASTM A 641 “ Standard Specification for Zinc-Coated (Galvanized) Carbon Steel Wire”

- ASTM A 653 “Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process”

- ASTM E 1477 “Standard Test Method for Luminous Reflectance Factor of Acoustical Materials by use of Integrating-Sphere Reflectometers”

  - SUBMITTALS

A.  Product Data: Manufacturer’s product data and installation instructions

B.	Shop Drawings:  Reflected ceiling plans coordinating penetrations and ceiling mounted items.  Show the following details:

a.  Details and reflected ceiling plans for the Clip-On installation

b.  Clearly illustrate all components of the Clip-On suspension and                                      components

  - System assembly details and connections to building components;

  - Location and direction of light fixtures, diffusers, speakers and other finish items

  - Framing and support details for work supported by acoustical ceiling suspension system

  - List of materials, dimensions, hanger fastenings and any special details

C.   Samples:  Minimum 12 inch (300 mm) piece of each type and finish with acoustical material and suspension components

D.  Certification:  Manufacturer’s certifications that products comply with specified requirements, including laboratory reports showing compliance with specified tests and standards

  - QUALITY ASSURANCE

- Manufacturer and Installer Qualifications: Provide Clip-On metal ceiling components produced by a single manufacturer with resources adequate to deliver a product of consistent quality in terms of appearance and physical properties for all project scopes and scales without risk of delay or interruption; Installation work to be performed by a firm whose personnel have no less than three (3) years of successful experience on projects of similar size, requirements and complexity

  - DELIVERY, STORAGE AND HANDLING

- Deliver materials in manufacturer’s unopened packages; suitably store to protect against exposure to moisture, sunlight, surface contamination and other unacceptable conditions

- Handle components to prevent damage

  - PROJECT CONDITIONS

- Environmental requirements at installation

  - The building shall be enclosed, the air conditioning system shall be operating with proper filters in place and the proper temperature and humidity conditions shall be stabilized before, during and following installation until Substantial Completion.  Building areas to receive ceilings shall be free of construction dust and debris

  - Coordination: Coordinate acoustical ceilings work with installers of related work including but not limited to building insulation, drywall, mechanical and electrical systems

- Dimensional Stability: Clip-On acoustical metal pan ceiling installation shall be carried out in temperature conditions up to 120°F (49°C), in spaces before the building is enclosed and HVAC is cycled or not operating

  - WARRANTY (LIMITED)

- Acoustical Panels: Submit a written warranty executed by the manufacture agreeing to repair or replacement of acoustical panels that fail within the warranty period.  Failures include but are not limited to:

  - Clip-On acoustical metal panels: Sagging, warping, rusting and manufacturer’s defects

  - Clip-On suspension: Rusting and manufacturer’s defects

- Warranty period for Clip-On panels and suspension is 1 years from date of Substantial Completion

1.09 MAINTENANCE

A. Provide extra acoustical panels and suspension material, matching installed material, in manufacturer’s original packages and clearly labeled as attic stock as specified in DELIVERY, STORAGE AND HANDLING section

B.  Deliver extra stock and access tools to owner’s representative

PART 2 PRODUCTS

## 2.01 MANUFACTURER

Provide a Clip-On system manufactured by Simplex Ceilings, 800.638.5726, info@simplexceilings.com

## MATERIALS

A.  Suspension System: Type 1

B.  Product:  (Simplex Ceilings Clip-On Suspension System)

C. Components:

  - Interior;

    - (Intermediate Duty)(Heavy Duty) main tees, cross tees and wall molding

    - Hanger wire, 12 gauge pre-stretched galvanized steel wire

b.  Exterior;

i.  Simplex Ceilings Clip-On Exterior Heavy Duty main tees, cross tees and wall molding

ii. Rigid vertical hangers to be designed according with drop length      and design loads

D. Finish: The 15/16” flange shall be (finished to match the panel color) (white) (black).  The exposed flange of the wall molding and fixture trim shall be finished to match the panels

2.03 MANUFACTURER

- Provide a Clip-On system manufactured by Simplex Ceilings, 800.638.5726, info@simplexceilings.com

  - MATERIALS

- Metal Ceiling: Type AP-1

- Type: Simplex Ceilings Clip-On Acoustical Metal Pan System – (unperforated)(perforated)(black acoustical textile)(insulation pads & spacers)

  - Perforations:

i.  Standard: (microperforated, pattern 2, 12% open, 0.0625” diameter hole, 0.226” OC, 45° staggered, 0.240” border) (extra microperforated, MicroPlex, pattern 13, 0.7% open, 0.020” diameter hole, 0.196” OC, straight row, 0.571” border)(unperforated)

ii. Optional:

1. (pattern 1, 5% open, 0.125” diameter hole, 0.5” OC, straight row, 0.242” border)

2. (pattern 3, 7% open, 0.0625” diameter hole, 0.297” OC, 45° staggered, 0.261” border)

3. (pattern 4, 22.7% open, 0.094” diameter hole, 0.188” OC,  60° staggered, 0.242” border)

4. (pattern 5, 10% open, 0.125” diameter hole, 1.5” OC, 45° staggered, 0.242” x 0.367” border)

5. (pattern 6, 20%  open, 0.125” diameter hole, 0.25” OC,   straight row, 0.242” border

6. (pattern 7, 40% open, 0.125” diameter hole, 0.25” OC, 45° staggered, 0.242” border)

      - (pattern 8, 45% open, 0.75” square hole, 1.125” OC, straight row, 3.18” border)

      - (pattern 9, 21% open, 0.56” diameter hole, 1.094” OC, straight row, 3.257” border)

      - (pattern 10, 16% open, 0.5” square hole, 1.25” OC, straight row, 3.492” border)

      - (pattern 11, 25% open, 0.0625” diameter hole, 0.157” OC, 45° staggered, 0.217” border)

      - (pattern 12, 42% open, 0.5” diameter hole, 0.972” OC, 45° staggered, 2.104” border)

      - (pattern 15, 11% open, 0.25” square hole, 0.75” OC, straight  row, 0.250” border)

      - (pattern 16, 11% open, 0.375” square hole, 1.125” o.c., straight row, 0.075” border)

      - (custom _____________)

  - Acoustical Insulation

    - Standard:

      - (black acoustical textile)

      - (1-1/2” x ¾ pcf, Class A, poly wrapped)

    - Optional:

      - (white acoustical textile)

      - (___” x ___pcf, Class A, poly wrapped)

  - Accessories

    - (Trim Frames shall be finished to match the Clip-On panels and installed at lay-in light fixtures and air diffusers)

    - Cutouts for down lights, sprinklers and speakers shall be (factory) (field) fabricated

    - (Perimeter trim shall be as indicated on the drawings)

  - Performance Characteristics:

    - Sound Control:

      - (NRC .90, microperforated, pattern 2 with 1-1/2” x ¾ pcf fiberglass)

      - (NRC .70, microperforated, pattern 2 with acoustical textile)

      - (NRC .65, extra microperforated, MicroPlex, pattern 13 with 1-1/2” x ¾ pcf fiberglass)

      - (NRC .50, extra microperforated, MicroPlex, pattern 13 with acoustical textile)

      - (NRC .45, extra microperforated, MicroPlex, pattern 13 without acoustical textile or fiberglass)

      - (NRC .10 NRC, unperforated)

    - Flame Spread

      - ≤25, per ASTM E 84

      - Class A, per ASTM E 1264

    - Smoke Developed

      - ≤50 per ASTM E 84

    - Light Reflectance

      - (LR = 78% for unperforated white [BWE] paint)

      - (LR = 72% for perforation pattern 1 with black acoustical textile and White [BWE] paint)

      - (LR = 63% for perforation pattern 2 with black acoustical textile and White [BWE] paint)

      - (LR = 67% for perforation pattern 5 with black acoustical textile and White [BWE] paint)

      - (LR = 62% for perforation pattern 6 with black acoustical textile and White [BWE] paint)

      - (LR = 48% for unperforated Silverlume [LSM] paint)

      - (LR = 80% for unperforated Satin [112] anodized)

      - (LR = 74% for perforation pattern 1 with black acoustical textile and Satin [112] anodized)

      - (LR = 67% for perforation pattern 2 with black acoustical textile and Satin [112] anodized)

      - (LR = 69% for perforation pattern 5 with black acoustical textile and Satin [112] anodized)

      - (LR = 64% for perforation pattern 6 with black acoustical textile and Satin [112] anodized)

      - (LR = 60% for perforation pattern 11 with black acoustical textile and Satin [112] anodized)

      - (LR = 48% for perforation pattern 5 with black acoustical textile and Silverlume [LSM] paint

      - (LR 1 of 89% for white [CeilTex] painted glass fabric)

    - Wind Load Design

      - (40 psf positive & negative [24” x 24” panels])

      - (custom _______________)

  - Surface Texture, Substrate, Size and Edge

    - Surface Texture: (perforated pattern___) (unperforated)

    - Surface Finish:

      - Standard: (polyester baked enamel)

      - Optional

        - (polyester powder)

        - (fluoropolymer)

        - (lacquer mill)

        - (anodized [bright] [brushed] [Brushalum] [satin])

        - (304 stainless [2D] [4] [6] [8] [embossed ___])

        - (CeilTex [painted glass fiber fabric])

        - (DecoPlex [graphic image transfers])

        - (VENEERS [wood veneer])

        - (LAMINATES [wood grain laminate])

        - (IMAGES [wood grain image transfers]

        - (custom ____)

  - Color

    - (Paint)

      - Standard: (White)(Silverlume)(Gun Metal)

      - Optional:

        - (selected from manufacturer’s standard colors)

        - (custom color _______________)

    - (polyester powder___)(fluoropolymer ___)(lacquer mill)(anodized ___)(stainless___)(CeilTex___)(DecoPlex___)(VENEERS___) (LAMINATES___)(IMAGES___)(custom ___) as indicated on finish schedule.

  - Substrate

    - Standard:

      - (Interior 24” x 24” 0.032” aluminum)

      - (Interior 24” x 24” 0.024” aluminum, extra microperforated, MicroPlex, pattern 13)

      - (Exterior 24” x 24” 0.040” aluminum)

    - Optional:

      - (aluminum up to 0.063”)

      - (electrogalvanized steel up to 16 ga. 0.062”)

      - (stainless steel up to 16 ga. 0.062”)

  - Size:

    - Interior

      - Standard: 24” x 24”

      - Optional:

        - (12” x 12”)

        - (12” x 24”)

        - (12” x 36”)

        - (12” x 48”)

        - (12” x 60”)

        - (24” x 48”)

        - (24” x 60”)

        - (24” x 72”)

        - (36” x 36”)

        - (48” x 48”)

        - (Custom ___x___)

    - Exterior:

      - Standard:

        - (24” x 24”)

        - (24” x 48”)

      - Optional:

        - (12” x 24”)

        - (12” x 36”)

        - (12” x 48”)

        - (custom___ x ___)

  - Edge Detail

    - Interior:

      - Standard: (square edge with ¼” reveal) (square butt edge)

      - Optional:

        - (square edge with 1/8” reveal)

    - Exterior:

      - Standard: (square edge with ¼” reveal)(square butt edge)

PART 3 EXECUTION

3.01	PREPARATION

- Examine construction and conditions under which system will be installed.  Do not proceed with installation until unsatisfactory conditions have been corrected

3.02	INSTALLATION

- All acoustical panels and suspension systems shall be installed in strict accordance with the manufacturer’s printed instructions and current recommendations and in compliance with ASTM C 636 and the governing code of jurisdiction

- Application Considerations: Cutouts and apertures available.  They are intended for use with audio speakers, air diffusers, sprinklers and certain light fixtures.  They are not intended for use with smoke detectors

- Installed panels should be free from damage or defects

- Install partial panels as shown on the reflected ceiling plans but not less than one-half full size

  - FIELD QUALITY CONTROL

- Deflection of any suspension component shall not exceed 1/360 of the span.

  - ADJUSTING AND CLEANING

- Clean acoustical ceiling including trim, edge moldings and suspension members pursuant to manufacturer’s recommendations.  Remove and replace damaged components that cannot be restored.

END OF SECTION 09 51 33