ALTRO LAVENCIA CLICK

SECTION 09657

LUXURY VINYL TILE

PART 1 GENERAL

  - SUMMARY

- Section Includes: This section includes labor, materials and other services necessary to complete luxury vinyl tile installation and accessories work. Conform with requirements of all Sections of Division 1, General Requirements, as it applies to the work of this Section.

- Related Sections:

- Section 03300 - Cast-in-Place Concrete: Concrete finishing.

- Section 06100 - Rough Carpentry: Plywood floor sheathing.

- Division 7 - Thermal and Moisture Protection.

- Division 15 - Mechanical.

1.2	REFERENCES

- ASTM C1028-96,  Standard Test Method for Determining the Static Coefficient of Friction of Ceramic and Other Like Surfaces by the Horizontal Dynamometer Pull-Meter Method.

- ASTM E 648/NFPA 253, Standard Test Method for Critical Radiant Flux of Floor-Covering Systems Using a Radiant Heat Energy Source.

- ASTM E662, Standard Test Method for Specific Optical Density of Smoke Generated by Solid Materials.

- ASTM F 970, Standard Test Method for Static Load Limit.

- ASTM F1514, Standard Test Method for Measuring Heat Stability of Resilient Vinyl Flooring by Colour Change

- ASTM F137-08, Test Method for Flexibility of Resilient Flooring Materials with Cylindrical Mandrel Apparatus

- (RFCI) Resilient Floor Covering Institute

1.	RFCI Standard Slab Moisture Test Method (Calcium Chloride Method) as a supplementary test method to ASTM F2170.

- Underwriters Laboratories of Canada (ULC)

1.	CAN/ULC-S102.2, Surface Burning Characteristics of Flooring, Floor Covering and Miscellaneous Materials and Assemblies

1.3	SUBMITTALS

- Product Data: Submit manufacturer’s current printed product literature, specifications, installation instructions, and field reports in accordance with Section 01330 - Submittal Procedures.

- Shop Drawings: Submit shop drawings to indicate materials, details, and accessories in accordance with Section 01330 - Submittal Procedures including but limited to the following:

- Submit a cut diagram indicating seam locations and roll direction. Use mitered seam layouts for corners when changing directions 180 degrees (e.g. when running material down corridors which bisect at a right angle), unless approved otherwise.

- Samples: Submit duplicate 6" x 9" (152 mm x 228 mm) sample pieces of sheet material, 12" (300 mm) long [gulley edge] [cap strip] [joint cover strip] [cove former] in accordance with Section 01330 - Submittal Procedures.

- Closeout Submittals: Submit the following:

- Operation and Maintenance Data: Submit manufacturer’s operation and maintenance data for incorporation into manual specified in accordance with Section 01780 – Closeout Submittals. Include methods for maintaining installed products and precautions against cleaning materials and methods detrimental to finishes and performance.

1.4	QUALITY ASSURANCE

- Installer Qualifications: Installer experienced in performing work of this section who has specialized in installation of work similar to that required for this project.

  - Training: Installer who has attended an Altro flooring installation training clinic.

- Mock-ups: Install at project site a job mock-up using acceptable products and manufacturer approved installation methods, including concrete substrate testing.

  - Maintenance: Maintain mock-up during construction for workmanship comparison; remove and legally dispose of mock-up when no longer required.

  - Incorporation: Mock-up may be incorporated into final construction upon Owner’s approval.

- Pre-installation Meeting: Conduct pre-installation meeting to verify project requirements, substrate conditions, manufacturer’s installation instructions, manufacturer’s warranty requirements, and installer qualifications.

1.5	SITE CONDITIONS

- Temperature Requirements: If storage temperature is below 65F (18C) or the floor temperature is below 50F (18C), the Altro Lavencia Click must be moved to a warmer place and allowed to reach this temperature before unrolling or installation. For further information, refer to current Altro Installation Practices and Quick Facts.

- Maintain air temperature and structural base temperature at flooring installation area between 68F (20C) and 80F (26C) for 48 hours before, during and 24 hours after installation.

1.6	DELIVERY, STORAGE AND HANDLING

- Ordering: Comply with manufacturer’s ordering instructions and lead time requirements to avoid construction delays.

- Deliver, store and handle resilient flooring materials in accordance with Section 01610 - Basic Material	 Requirements.

- Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

- Store materials protected from exposure to harmful weather conditions, at temperature and humidity conditions recommended by manufacturer.

- Store rolls in dry locations. Stand rolls on end. Protect and secure rolls from falling.

1.7	WASTE MANAGEMENT AND DISPOSAL

.1	Deposit all packaging materials in appropriate container on site for recycling or reuse.

.2	Avoid using landfill waste disposal procedures when recycling facilities are available.

.3	Keep all discarded packaging away from children.

1.8	WARRANTY

- Warranty period for Altro Lavencia Click shall be 20 years commencing on date of substantial completion.

PART 2	 PRODUCTS

2.1	Luxury Vinyl Tile

- Vinyl Tile Manufacturer:  Lavencia Click by Altro,

- CANADA: 6221 Kennedy Rd, Unit 1, Mississauga ON, L5T 2S8

Toll Free: 800.565.4658 Tel: 905.564.1330 Fax: 905.564.0750

- .2.	Acceptable material: Altro Lavencia Click (measurements and product weights listed vary based on colour selected and are approximate);

- Including but not limited to:

7” x 48”, 5mm gauge, 22 mil wear layer, 8 pieces /ctn, 18.91 ft2 /ctn, 38.24 lbs /ctn

12” x 24”, 5mm gauge, 22 mil wear layer, 8 pieces /ctn, 16 ft2 /ctn, 32.38 lbs /ctn

9.65” x 59.9”, 5mm gauge, 22 mil wear layer, 6 pieces /ctn, 24.08 ft2 /ctn, 49.2 lbs /ctn

COLOUR

Colour to be selected from current range found at www.altrofloors.com

# 1.  LAAC_ (insert Alpha-Numeric colour code) (insert colour name)

2.2	ACCESSORIES

Including but not limited to:

- .1	Subfloor Filler and Leveler: Use only gray Portland cement-based “moisture tolerant” underlayments, and patching compounds. Use for filling cracks, holes or leveling. White gypsum materials are not acceptable.

- .2	Adhesives:

  - Ecofix 25 – 1 part acrylic for areas free of moisture and/or temperature fluctuations.

  - Ecofix 20 – 1 part acrylic for areas free of moisture with heavy rolling loads.

PART 3	 EXECUTION

3.1	EXAMINATION

- Compliance: Comply with manufacturer’s product data, including product technical bulletins, product catalog, installation instructions.

- Site Verification of Conditions: Verify substrate conditions, which have been previously installed under other sections, are acceptable for product installation in accordance with manufacturer’s instructions.

3.2	PREPARATION

  - Altro flooring shall be installed over subfloors conforming to ASTM F710 for concrete and other monolithic floors or ASTM F1482 for wood subfloors.

  - Always conduct moisture tests per ASTM F-2170 on all concrete slabs regardless of age or grade level. ASTM F-2170 Internal Relative Humidity (IRH) test results must not exceed 85%.

  - Do not proceed with work until results of moisture condition tests are acceptable.

  - When patching, a moisture tolerant patching compound must always be used.

3.3	INSTALLATION

- Installation: Install Altro Lavencia Click in accordance with the current posted Altro Installation Practices at www.altrofloors.com, Downloads, Technical Documents, Installation Guides. All Seams shall be heat welded with Altro Weldrod™ only. Failure to install Altro Lavencia Click flooring in accordance with recommended procedures will void the Altro Limited Product Warranty.

3.4	CLEANING

Specifier Note: Altro Lavencia Click is unaffected by surface water and most chemicals which do not have a solvent action on vinyl. Certain organic solvents and chemicals, including asphalt, can cause staining, as might some antioxidants found in certain types of rubber used in mats, wheels and tires.  Acids and dyes may affect the colour, which should be selected accordingly. Contact Altro for information about the effect of chemicals on Altro flooring.

- Cleaning: Remove temporary coverings and protection of adjacent work areas.

- Repair or replace damaged installed products.

- Clean installed products in accordance with manufacturer’s instructions prior to Owner’s acceptance.

- Current recommended maintenance procedures can be found on the Altro website at www.altrofloors.com, Downloads, Technical Documents, Maintenance Guides.

- .2   Protection:

- Sweep or vacuum all construction debris and dust first, then clean the flooring with AltroClean 44 /AltroClean 44 Plus using an auto scrubber.

- Cover and protect finished installation from damage from other trades using a non-staining, temporary floor protection system, such as reusable textured plastic sheeting.

- Never use tapes on the surface on the finish flooring, Sharpies, pens, crayons or construction markers on either the finish flooring or the substrate.

- No traffic for 24 hours after installation, unless approved by Altro technical.

- No heavy traffic, rolling loads, or furniture placement for 72 hours after installation.

- Wait 72 hours after installation before performing initial cleaning.  Start a regular maintenance program after the initial cleaning as recommended by manufacturer

3.5	PROTECTION

- Cover and protect finished installation from damage from other trades using a non-staining, temporary floor protection system, such as a reusable textured plastic sheeting.

- Altro Lavencia Click should be covered and protected from all other trades during construction with a suitable non-staining protective covering without taping to the surface of the flooring.

END OF SECTION