[image: image1.jpg]decoustics

SAINT-GOBAIN




Rondolo™ Acoustical Micro Perforated Wood Panels 

"Direct to Substrate”
PART 1 GENERAL

1.01 Scope
A. Include all labor, materials, and plant as required for the fabrication and delivery of acoustical wall system complete in all respects as described and specified herein. 

1.02 Related Work by Other
A. Gypsum board partition and wall structure

B. Paint finish of wall surface behind and between panelling

C. Lighting fixtures and electrical services and connections

D. Other acoustical products  

1.03 Quality Assurance
A. Manufacturer & Installer: Firm manufacturing the specified product shall have adequate capacity required for projects listed and have successfully completed similar projects for a period of not less than five years. The installer should be approved by the manufacturer as qualified to perform work required.

B. Reference Standards: Conform to all governing laws, building codes, and the following performance criteria:

C. Fire Performance Characteristics: Provide Veneer Finished Mini/Micro perforated panels with surface‑burning characteristics as determined by testing panel components in accordance with ASTM E84 test procedures. 

ASTM E‑84 testing must be performed by an independent testing organization acceptable to authorities having jurisdiction. 





    ATM E-84 Classification

Class “A” or “1”

                                                           Flame Spread:
25 or Less

                                                    Smoke Developed:
450 or Less

1. Acoustical Performance Characteristics:  Provide Micro-perforated wood panels with acoustical absorption characteristics as indicated in Part 2, which have been determined by testing fully assembled production material with no insulation in accordance with ASTM C‑423 (Type F5 and F25 mounting method as defined by ASTME-795) by a testing organization acceptable to authorities having jurisdiction
1.04 Submittals
A. Submit to the Architect of record, three (3) complete sets of CAD generated shop drawings prepared by the manufacturer showing all necessary details and dimension requirements which will subsequently be field verified and revised as required by the Architect.

B. Samples: Submit (3 sets) manufacturers standard 8" X 11" sample panels of each type of product as specified in Part 2 to the Owner for approval.  Product shall be original production material in veneer finish specified for final use.

C. Certification: Submit to the owner a certificate of compliance to specified acoustical and fire performance criteria as stated section 1.03 and Part 2 of this specification, signed by an officer of the panel manufacturer and attach independent laboratory test results for each product used, showing that the products supplied as components meet or exceed the specified requirements.  Submit additional test results to owner as requested detailing compliance to updated code requirements.

D. Prior Approval:  Manufacturers not listed in section 2.01 of this specification and wishing to be submitted as an equal on the project must submit information in accordance with section 1.04 of this specification fifteen days prior to the bid date as outlined in the contract documents.  The information submitted must give evidence to show that the alternative product equals or exceeds the attributes and performance of the specified product.

E. Manufacturers Approval: The manufacturer shall have the right to approve the selection of the installing contractor and to verify that said contractor has sufficient experience and expertise to complete the project in a satisfactory manner.

F. Single Source: It is the clear intent of this specification to provide a complete, fully integrated system, supplied by a single company. “Stick built” parts and pieces from various and different manufacturers will not be accepted. All custom acoustical wall and ceiling panels shall be purchased from a single supplier
1.05 Product Delivery, Storage and Handling
A. Deliver Micro-Perforated Wood Panels and related components to the site for installation in accordance with a reasonable schedule furnished by the contractor.  On‑site storage shall be such as to assure that all panels and associated materials are protected from damage.

B. Prior to panel installation the site must be free of all wet and dusty trades and the climatic conditions stabilized to normal operational levels. Panels shall be allowed to stabilize on site 72 hours prior to installation.

C. PANELS MUST BE STORED, INSTALLED AND MAINTAINED ONLY IN A SECURE AMBIENT ENVIRONMENT (HUMIDITY MIN. 35% - MAX. 55%, TEMPERATURE NOT TO EXCEED 80º F (27º C)).
1.06 Guarantee
A. Furnish to the Architect in the Owner's name, the manufacturers written guarantee covering the products supplied against defects in materials and workmanship under normal operating conditions for a period of one year from the date of shipment. Submit certificates of compliance showing warranty period by dates for each project completed to the Owner. 

B. All system components shall be manufactured and supplied by the named manufacturer. Individual components, provided by individual suppliers will not be considered or accepted. 
PART 2 PRODUCTS
2.01
Rondolo™ Acoustical Micro Perforated Acoustic Wall Panels


Furnish and deliver Prefabricated Decoustics Rondolo™ Acoustical Micro Perforated Wood Panels as described in this section for installation in areas as shown on drawings meeting or exceeding the following requirements:


Decoustics 


61 Royal Group Crescent,
 TEL: (800) 387-3809  TEL: (905) 652-5200



Woodbridge, Ontario.
 FAX: (905) 652-2505


Canada, L4H 1X9.

 Email: sales@decoustics.com
1. Micro Perforated Rondolo™ acoustic wall and ceiling panels shall be MiP16 Type 1/8 comprised of a single piece Class A fire rated “No Added Urea Formaldehyde” MDF core of 16mm (5/8”) thick faced with a natural wood veneer laminated to the panel face and an acoustic black matt laminated to the back side of the panel. Perforations are a nominal 0.55mm and spaced 1.7mm horizontally and 1mm vertically apart. The structure is perforated as shown on the architectural and approved shop drawings. The panels shall be fabricated to sizes required by field dimensions supplied by the installing contractor and will not vary from determined sizes by more than +/- 3 mm (1/8") vertically and horizontally. The panel shall be constructed using fire retardant core components meeting or exceeding Class A (Flame-spread of 25 or less and Smoke Developed of 50 or less) as a composite finished product when tested in accordance with ASTM E84 procedures. The panel shall be constructed using fire retardant, no added urea formaldehyde, core components. The panels shall be installed utilizing Decoustics Direct Mount Wall System.
Veneer to be:


(A)
Select veneer from Manufacturers standard range of lacquer finished real wood veneers.









OR

B)
Architect / Designer specified finish
Source and laminate Architects specified veneer species, stained and lacquered to match Architects sample.
Unless otherwise specified, veneer will be quarter cut, slip matched. Veneer will not be sequenced.  Clear lacquer finish to be 30% sheen.
2. Installation shall be by use of concealed aluminum panel clips factory supplied, and field attached to the back of the panels, engaged into factory supplied wall clips or continuous Z track fastened to the wall. Clip spacing to be maximum horizontal centres of 24" (610mm) (adhesively mounted clips are not acceptable).
3. Installation shall be in accordance with the manufacturers’ instructions and as shown on Decoustics shop drawings and/or details. Installer shall provide shimming and adjustments as required to maintain consistent alignment of joints and of finished panel faces.

4. Panels shall have noise reduction coefficient values of the following when tested in accordance with section 1.03 B 2 of this specification.








Frequency (Hz)

Description

Thickness
    125
   250
  500
 1000
2000
4000
NRC
SAA

Rondolo ™ 

16 mm (5/8")     0.07   0.09
  0.22
 0.60
0.96
0.38
0.45
0.46
(No fiber glass backing behind panels)
Rondolo ™ 

16 mm (5/8")     0.26   0.51
  1.10
 1.01
0.85
0.54
0.85
0.87
(1” thick fiber glass backing behind panels)
PART 3 INSPECTIONS AND APPROVAL
1.01 Adjustment and Replacement
A. The Owner shall inspect the installation and product on completion.  The manufacturer shall provide repair or replacement of components not conforming to requirements as stated herein and said work will then become bound by the terms of this specification. 

B. Installation labor for removal and replacement of product improperly installed and not conforming to specified installation instructions as detailed in section 1.03 and Part 2 and shown on plans, shall be the responsibility of the installing Contractor.

C. Rondolo™ Micro-Perforated panels are a real wood veneer product. Every effort is made to maintain the overall appearance; however, natural variations in grain, texture, shade and aging may occur in varying site conditions. For these reasons, the manufacturer cannot guarantee the exact matching of grain, pattern and color.

Decoustics "Wall Mount” System
Rondolo™ Acoustical Micro Perforated Wood Panels 

December 2016
PAGE  
3
Decoustics "Wall Mount” System
Rondolo™ Acoustical Micro Perforated Wood Panels 

December 2016

