decoustics





 Decoustics LightFrame
Translucent Acoustic Ceiling Panels 

PART 1. GENERAL
1.1 Scope
A.
Include all labor, materials, and plant as required for the fabrication and delivery of acoustical ceiling panels for the system described and specified herein. 

1.2 Related Work by Other
A.
Gypsum board partition, wall and ceiling structure, perimeter and intermediate structural sub-frame(s) for Translucent Acoustic Ceiling Panels.

B.
Paint finish of ceiling surface behind and between panelling.

C.
Lighting fixtures and electrical services and connections.

D.
Air handling, heating and air conditioning, plumbing and other mechanical services as required.

E.
Public address system, speakers, wiring, support and installation.


F.
Life safety equipment.


G.
Ceilings by others not included in this scope.

1.3 References


A.
ASTM E-84 Standard Test Method for Surface Burning Characteristics of Building Materials.

B.
ASTM C423 Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

C.
ASTM E795 Standard Practices for Mounting Test Specimens During Sound Absorption Tests.
D.
ISO 354 Acoustics – Measurement of Sound Absorption in a Reverberation Room.
1.4 Submittals
A.
Submit to the Architect of record a complete set of CAD generated shop drawings prepared by the manufacturer showing all necessary details and dimension requirements which will subsequently be field verified and revised as required by the Architect.

B.
Samples: Submit 1 manufacturers’ standard 200mm (8") x 200mm (8") sample panel of the product as specified in Part 2 to the Owner for approval.  Product shall be original production material in fabric finish specified for final use.

C.
Certification: Submit to the owner a certificate of compliance to specified acoustical and fire performance criteria as stated section 1.03 and Part 2 of this specification, signed by an officer of the panel manufacturer and attach independent laboratory test results for each product used, showing that the products supplied as components and complete assemblies, meet or exceed the specified requirements..

D.
Prior Approval:  Manufacturers not listed in section 2.01 of this specification and wishing to be submitted as an equal on the project must submit information in accordance with section 1.04 of this specification fifteen days prior to the bid date as outlined in the contract documents.  The information submitted must give evidence to show that the alternative product equals or exceeds the attributes and performance of the specified product.

E.
Manufacturers Approval:  The manufacturer shall have the right to approve the selection of the installing contractor and to verify that said contractor has sufficient experience and expertise to complete the project in a satisfactory manner.

F.
Single Source: It is the clear intent of this specification to provide a complete, fully integrated system, supplied by a single company. “Stick built” parts and pieces from various and different manufacturers will not be accepted. All custom acoustical wall and ceiling panels shall be purchased from a single supplier.

1.5 Quality Assurance
A.
Manufacturer & Installer:  Firm manufacturing the specified product shall have adequate capacity required for projects listed and have successfully completed similar projects.  The Installer should be approved by the manufacturer as qualified to perform work required.

B.
Reference Standards:  Conform to all governing laws, building codes, and the following performance criteria:

Fire Performance Characteristics: Provide ceiling panels with surface‑burning characteristics as determined by testing panel assemblies in accordance with ASTM E84 test procedures. Testing must be performed by a testing organization acceptable to authorities having jurisdiction.


ASTM E-84

Classification

      Class "A" or "1"



Flame Spread:

25 or less





Smoke Developed:

50 or less


C.
Acoustical Performance Characteristics:  Provide ceiling panels with 


acoustical absorption characteristics as indicated in Part 2, which have been 

determined by testing fully assembled production material in accordance with 

ASTM C‑423 (Type "E400" mounting as defined by ASTM E-795) by a testing 

organization acceptable to authorities having jurisdiction.  Approved testing 

organization must be independent of the manufacturer.

D.
Decoustics LightFrame panels shall comply with the requirements of the “Standard method for the testing and evaluation of volatile organic chemical emissions from indoor sources using environmental chambers version 1.1)” as defined by the California Department of Public Health (CDPH) – Version of February 2010 (Reference: California Specification Section 01350).

1.6  Product Delivery, Storage and Handling
A.
Deliver fabricated units and related components to the site for installation in accordance with a reasonable schedule furnished by the contractor.  On‑site storage shall be such as to assure that all panels and associated materials are protected from damage, and storage area is climatically controlled to normal operational levels..

B.      Prior to panel installation, the site must be free of all wet and dusty trades and 
the climatic conditions stabilized to normal operational levels.  Panels shall be 
allowed to stabilize on site 24 hours prior to installation.

C.      Panels must only be handled by persons wearing clean light-weight gloves. It 
is very important that personnel installing hardware (clips, ceiling suspension 
members/systems, springs etc.) do not handle the panels before putting the 
clean lightweight gloves on.  

1.7  Guarantee
A.
Furnish to the Architect in the Owner's name, the manufacturers written guarantee covering the products supplied against defects in materials and workmanship under normal operating conditions for a period of one year from the date of shipment.  Submit certificates of compliance showing warranty period by dates for each project completed to the Owner. 

PART 2 PRODUCTS

2.1 Decoustics LightFrame Translucent Acoustical Ceiling Panel
Furnish and deliver prefabricated translucent acoustical ceiling panels as described in this section for installation in areas as shown on drawings 
A. Description: The ceiling system shall be the "Decoustics LightFrame” system as manufactured by:-

Decoustics


 61 Royal Group Crescent


TEL: (800)387-3809



 Woodbridge, Ontario


FAX: (905)652-2505



 Canada, L4H 1X9



Email: sales@decoustics.com

Decoustics LightFrame panels shall be type LF with a concealed extruded aluminum frame. The frame shall incorporate the fabric mounting mechanism and hardware to mount the panels into a structural sub-frame provided by the installing contractor.  The panel shall be free of surface defects and of a uniform thickness which will not vary by more than +/- 1.0mm (0.03"). The panels shall be fabricated to sizes supplied by the installing contractor to a tolerance of +/- 1.6mm (0.06"). Concealed aluminum edges shall be straight and true, and must be in alignment with the panel face. The edges of all panels shall be clearly marked with the project I.D. number, panel number, location code, and correspond to the shop drawings. The panels shall be installed into the customer supplied perimeter and intermediate (where applicable) sub frames. The panels shall be completely engineered and fabricated in the factory, to avoid any field cutting, and manufactured as shown on the architectural and approved shop drawings. 

B.
Finish shall be:



SEFAR Architectural (PVDF) Fabric and ETFE foil – acoustically transparent incorporating high light transmission. 




Fabric shall be applied over the face and the ETFE foil applied on the rear side of the panels, and held securely in place by the integrated frame mounting system (Note: PVDF fabric may be applied on the rear side of the panel in lieu of the ETFE foil if required – this option would be priced accordingly).
C.
Installation shall be by use of the standard “cog-wheel bolt” or “perimeter bolt” supplied with the panels. Special tools shall be supplied with the panels for installing the panels and for future access of the panels. The “panels are lifted into place and the “cog-wheel bolts” or “perimeter bolts” are engaged into the installer supplied perimeter or intermediate sub frame, or to adjacent panels. The “cog-wheel bolts” are tightened using the tool supplied. For further information, refer to the “Cleaning and Maintenance Instructions” provided for the project. (Note: An installer supplied structural perimeter frame is required for installing Decoustics LightFrame Panels.)
D.
Installation shall be in accordance with local code requirements, manufacturers’ instructions, and as shown on Decoustics approved shop drawings. Installer shall provide for adjustments as required to maintain consistent alignment of panels and of finished panel faces, and to ensure unstressed mounting locations.

E.
Panels shall have noise reduction coefficient values of the following when tested in accordance with section 1.5C of this specification.

Panel






Frequency (Hz)

Type
       Thickness
Finish     125     250
 500
1000
2000
4000
NRC
SAA

LightFrame  80mm ( 3 1/8”)    IA80CL   0.91   0.79
 0..53
0.77
0.79
0.75
 0.70
 0.67
0.1mm ETFE Film on back

LightFrame  80mm ( 3 1/8”)    IA85OP   0.78  0.55   0.47    0.80
0.64
0.61
 0.60
 0.57
0.1mm ETFE Film on back

PART 3  INSPECTION AND APPROVAL

3.1  Adjustment and Replacement
A.
The Owner shall inspect the installation and product on completion.  The manufacturer shall provide repair or replacement of components not conforming to requirements as stated herein, and said work will then become bound by the terms of this specification. 

B.
Installation labor for removal and replacement of product improperly installed and not conforming to specified installation methods as detailed in section 2.1.3 and 2.1.4, and shown on approved shop drawings, shall be the responsibility of the installing Contractor. 
1
Decoustics LightFrame

Translucent Acoustic Ceiling Panels

February 2014 


