SECTION 09 51 33 – USG PANZ™ METAL CEILINGS

Product Summary:

- Simple lay-in design provides 100% access. - Acoustibond™ sound backer produces .65 NRC (pattern dependent) without additional acoustical material when panels are installed within an enclosed plenum (minimum plenum depth: 18"). - Available in numerous perforation patterns. - Creates reveal edge on field-cut panel. - Available with same edge details as USG acoustical panels. - Panel thickness: Reference table below for base metal thickness. - <insert product representative here> or 800.874.4968 for technical questions.

| panel size | 12x48 | 12x60 | 24x24 | 24x48 | 24x60 | 24x72 | 48x48 | 30x30 | 30x60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| thickness SI | 0.021 | 0.021 | 0.021 | 0.032 | 0.032 | 0.032 | 0.040 | 0.032 | 0.040 |
| thickness MM | 0.5334 | 0.5334 | 0.5334 | 0.8128 | 0.8128 | 0.8128 | 1.016 | 0.8128 | 1.016 |
| Note: All metal thickness to be uniform within a given project area/space, typical. |  |  |  |  |  |  |  |  |  |

Note to Specifier:

This document is not intended to function as a standalone specification. It is intended to assist the specifier in inserting the proper language into the following recommended specification sections: 09 51 33.

- 2.3   USG PANZ™ ALUMINUM METAL PAN CEILING TILES

Acoustical Panel Type <INSERT DESIGNATION HERE>:

Basis of Design: Subject to compliance with project requirements, the design is based on the following: USG Interiors, LLC, “Panz”.

Classification:  Provide ceiling panels complying with ASTM E 1264 for type, form and pattern as follows:

Type: VII, Perforated aluminum facing (pan) with mineral glass or fiber base backing.

Pattern: [A Perforated, round holes in a 45 degree configuration] [B Perforated round holes in a 60 degree configuration]  [C Perforated round holes in a straight configuration] [D Perforated square holes in various configurations] [A062] [A062A] [A062D] [A116] [A188] [A250] [B079] [B125] [B188A] [B197] [B250] [B250H] [B312] [B500] [C030] [C040] [C062] [C062A] [C116] [C188] [C250A] [C375] [C500] [D250] [D375] [D500] [D500D] [Select from List of Manufacturer’s Standard Patterns] [Reference Architectural Drawings] [Reference Material Schedule].

Pan Face Finish painted in color to match manufacturer’s full range: [Flat White 050] [Silver Satin 002] [Matte White 3708] [Metallic Copper 1690] [Metallic Gold 1691] [Metallic Oyster 1652] [Aged Copper 1017] [Etched Ice Dark 1016] [Etched Ice Light 1015] [Beech 3467] [Dark Bamboo 3465] [Dark Cherry 3468] [Light Bamboo 3466] [Light Cherry 3469] [Maple 3470] [Red Oak 3471] [Walnut 3472].

LR: Not less than [Flat White 050 without perforations 0.8]. [consult USG for values – it will vary depending on paint color and perforation pattern].

NRC: Not less than [0.65with acoustibond backer] [0.80with x-tech backer] [0.90with fiberglass backer].

CAC: Not less than [20] [30] [35].

Edge/Joint Detail: [FL] [SL] [SQ] [FM].

Metal Thickness: <as noted in schedule above>.

Suspension Grid Width [9/16 inch (14 mm)] [15/16 inch (24 mm)]

Panel Depth: [3/8 inch (9.5 mm)] [1/2 inch (12.7 mm)].

Modular Size: [12 by 48 inches (305 by 1219 mm)] [12 by 60 inches (305 by 1524 mm)] [24 by 24 inches (610 by 610 mm)] [24 by 48 inches (610 by 1219 mm)] [24 by 60 inches (610 by 1829 mm)] [24 by 72 inches (610 by 2438 mm)] [48 by 48 inches (1219 by 1219 mm)] [20 by 60 inches (508 by 1524 mm)] [30 by 30 inches (762 by 762 mm)] [30 by 60 inches (762 by 1524 mm)] [As indicated on Drawings] [As indicated in a schedule].

Recycled Content: [insert project requirements here %].

Required accessories: Stabilizer bars for all panels longer than 4’-11” (1500 mm)

Panel Warranty: 1-year USG Specialty Systems Warranty, reference USG SC2102

Accessories.

Add clips and items here – may need input from CSG/Faris for what we should consider including here.

M9 wall molding: 15/16” x 9/16 x 12’ (24 x 14 x 3658) long angle shape of pre-painted steel.

MS274 shadow molding: 7/8” x ¾” x 9/32” x 1-1/4” (22 x 19 x 7 x 32) for seismic installation.

PZTSL Panz Shadowline perimeter trim: ¼” x 5/8” x ½” x 12’ (6 x 16 x 12.7 x 3658).

PZTFL Panz Fineline perimeter trim: ¼” x 5/16” x ½” x 12’ (6 x 8 x 12.7 x 3658).

Panels with cutouts: For lighting, sprinklers, or speakers with integral trim to conceal the edge of the panel cutout (specify cutout sizes and locations for each type) or specify that cutouts are to be made in the field.

Suspension System Attachment devices.

Hanger Wire: Galvanized carbon steel; soft temper; pre-stretched; yield stress load at least three times the design load but not less than 12-gauge.

Spacing and gauge per IBC, UL and CISCA design.

Supplied and installed by ceilings subcontractor.

Disclaimer:  The USG Product Specifications contained herein are intended for use as product reference material by architects, engineers, other design professionals, contractors, building code officials, or other competent construction industry trade professionals having an interest in the selection, specification and use of products manufactured by the subsidiaries of USG Corporation. The Specifications are intended solely as technical support incident to the sale and use of USG products and not intended to be a substitute for the design review and approval of the licensed design professionals for the project.  These materials may be printed and/or transferred electronically solely as needed by the user. Because electronic text files can be modified by other parties, without notice or indication of such modifications, modification of USG Product Guide Specifications and Drawings is the sole responsibility of the Design Professional. Consult with your local USG product representative for current market values for the recycled content percentages.