SECTION 09 51 33 – mETAL CEILINGS - USG PARALINE™ BAFFLES

Sustainability Notes:

- High light-reflective finish (LR-0.90) for Matte White, 3708.

Product Summary:

- Specially engineered hardware allows for simple suspension. - Exceptional noise reduction with absorption - Concealed suspension components with no visible connection hardware. - Innovative connection options allow for grouped and ungrouped designs. - Unconstrained by a linked suspension system to allow freedom of assembly and alignment using independent suspension option - <Insert product representative here> or 800.874.4968 for technical questions.

Note to Specifier:

- This document is not intended to function as a standalone specification. It is intended to assist the specifier in inserting the proper language into the following recommended specification sections: 09 51 33 METAL CEILINGS. - Reference USG Literature IC631 and IC633 for more information.

- 2.3   USG PARALINE BAFFLES LINEAR METAL CEILING

Acoustical Panel Type <INSERT DESIGNATION HERE>:

Basis of Design: Subject to compliance with project requirements, the design is based on the following: USG Corporation, “Paraline Baffles”.

Classification:  Provide ceiling panels complying with ASTM E 1264 for type, form and pattern as follows:

Type: VII, Perforated aluminum facing (baffle) with mineral glass or fiber base backing.

Pattern: [A Perforated, round holes in a 45 degree configuration] [B Perforated round holes in a 60 degree configuration] [C Perforated round holes in a straight configuration] [D Perforated square holes in various configurations] [A062]  [A062F] [A116] [C030] [C040] [C062] [C116] [C375] [Select from List of Manufacturer’s Standard Patterns] [Reference Architectural Drawings] [Reference Material Schedule].

Pan Face Finish painted in color to match manufacturer’s full range: [White 050] [Silver Satin 002] [Matte White 3708] [Beech 3467] [Dark Bamboo 3465] [Dark Cherry 3468] [Light Bamboo 3466] [Light Cherry 3469] [Maple 3470] [Red Oak 3471] [Walnut 3472] [Brushed Aluminum 058] [Satin Chrome PM614].

LR: Not less than 0.9 with Matte White 3708 (without perforations).

Acoustical Backer: [None] [Acoustibond] [Polybag with acoustical insulation].

CAC: Not less than N/A.

Edge Detail: N/A

Baffle Thickness: 1 1/2 in (38 mm).

Baffle Height: [4 in (102 mm)] [6 in (152 mm)].

Baffle Length: [24 in (610 mm)] [48 in (1219 mm)]  [72 in (1828 mm)] [ 96 inches (2438 mm)] [120 in (3048 mm)] [144 in (3658 mm)] [As indicated on Drawings] [As indicated in a schedule].

Recycled Content: [enter value]

Panel Warranty: 1-year USG Specialty Systems Warranty, reference USG SC2102

- ACCESSORIES

          - Suspension Clip (1 per 4 Linear Feet typical + one at each end ) [Grid Suspension Clip] [Curvatura Suspension Clip] [Independent Suspension Clip]

Baffle Splice (one per baffle).

Baffle End Cap

Suspension System Hanger Wire: Galvanized carbon steel; soft temper; pre-stretched; yield stress load at least three times the design load but not less than 12-gauge.

Spacing and gauge per IBC, UL and CISCA design.

Supplied and installed by ceilings subcontractor.

Disclaimer:  The USG Product Specifications contained herein are intended for use as product reference material by architects, engineers, other design professionals, contractors, building code officials, or other competent construction industry trade professionals having an interest in the selection, specification and use of products manufactured by the subsidiaries of USG Corporation. The Specifications are intended solely as technical support incident to the sale and use of USG products and not intended to be a substitute for the design review and approval of the licensed design professionals for the project.  These materials may be printed and/or transferred electronically solely as needed by the user. Because electronic text files can be modified by other parties, without notice or indication of such modifications, modification of USG Product Guide Specifications and Drawings is the sole responsibility of the Design Professional.