SECTION 095133 – USG Celebration™ Snap-In Metal Ceilings

Product Summary:

- Aluminum panels provide a monolithic appearance. - Easy installation into standard USG Donn® Brand Fineline® DXF™ Acoustical Suspension System. - Aluminum panels are positively attached to the suspension system through engineered connection. - Downward panel access is excellent for shallow plenum areas. - Wall-mount solution available; see IC605. - Wind load tested and approved, including Miami-Dade NOA, for exterior applications with proper bracing and framing. - Panel thickness: Reference table below for base metal thickness. - Since panels are installed from below, the clearance for installation and removal of panels is minimal. Additional plenum clearance of up to 9" may be required for air boot and lighting installations, depending on the unit selected. - <insert product representative here> or call 800.874.4968 for technical questions.

| panel size | 12x48 | 12x60 | 24x24 | 24x48 | 24x60 | 24x72 | 48x48 | 30x30 | 30x60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| thickness SI | 0.021 | 0.021 | 0.021 | 0.032 | 0.032 | 0.032 | 0.040 | 0.032 | 0.040 |
| thickness MM | 0.5334 | 0.5334 | 0.5334 | 0.8128 | 0.8128 | 0.8128 | 1.016 | 0.8128 | 1.016 |

Note: All metal thickness to be uniform within a given project area/space, typical.

Note to Specifier:

- This document is not intended to function as a standalone specification. It is intended to assist the specifier in inserting the proper language into the following recommended specification sections: 09 51 33 ACOUSTICAL METAL PAN CEILINGS. - Panel lengths equal to or greater than 59 inches then one stabilizer bar or cross tee is required at midpoint. If panel length is ≥ 96 inches then two stabilizer bars or cross tees are required at the 1/3 and 2/3 points. - Cross tees ≥ 96 inches used in large modules shall have hanger wires installed minimum 4 ft. o.c. 3. - The perforation pattern and acoustical backer will impact acoustical performance. - Refer to the following USG literature for more information: - System brochure: IC415 - Data sheet: IC308 - Perforations selection guide: IC425 - Exterior ceiling systems guide: SC2561

- 2.3   USG CELEBRATION ALUMINUM METAL SNAP-IN CEILING TILES

Acoustical Panel Type <INSERT DESIGNATION HERE>:

Basis of Design: Subject to compliance with project requirements, the design is based on the following: USG Interiors, LLC, “Celebration”.

Classification:  Provide ceiling panels complying with ASTM E 1264 for type, form and pattern as follows:

Type: VII, Perforated aluminum facing (pan) with mineral glass or fiber base backing.

Pattern: [A Perforated, round holes in a 45 degree configuration] [B Perforated round holes in a 60 degree configuration]  [C Perforated round holes in a straight configuration] [D Perforated square holes in various configurations] [A062] [A062A] [A062D] [A116] [A188] [A250] [B079] [B125] [B188A] [B197] [B250] [B250H] [B312] [B500] [C030] [C040] [C062] [C062A] [C116] [C188] [C250A] [C375] [C500] [D250] [D375] [D500] [D500D] [Select from List of Manufacturer’s Standard Patterns] [Reference Architectural Drawings] [Reference Material Schedule].

Embossing Pattern: [EC25D] [EC25U] [EM25D] [EM25U].

Pan Face Finish painted in color to match manufacturer’s full range: [Flat White 050] [Silver Satin 002] [Matte White 3708] [Metallic Copper 1690] [Metallic Gold 1691] [Metallic Oyster 1652] [Aged Copper 1017] [Etched Ice Dark 1016] [Etched Ice Light 1015] [Beech 3467] [Dark Bamboo 3465] [Dark Cherry 3468] [Light Bamboo 3466] [Light Cherry 3469] [Maple 3470] [Red Oak 3471] [Walnut 3472].

LR: Not less than [Flat White 050 without perforations 0.8].

NRC: Not less than [0.60 A062 perforation with acoustibond backer] [0.65] [0.70 A116 perforation with acoustibond backer] [0.85 withA062 and polybag fiberglass backer] [0.90 withA116 and polybag fiberglass backer]..

Edge/Joint Detail: 1/8” Flush reveal.

Metal Thickness: <as noted in schedule above>.

Suspension Grid Width 9/16 inch (14 mm).

Modular Size: [12 by 48 inches (305 by 1219 mm)] [12 by 60 inches (305 by 1524 mm)] [24 by 24 inches (610 by 610 mm)] [24 by 48 inches (610 by 1219 mm)] [24 by 72 inches (610 by 1828 mm)] [24 by 96 inches (610 by 2438 mm)] [48 by 48 inches (1219 by 1219 mm)] [20 by 60 inches (508 by 1524 mm)] [30 by 30 inches (762 by 762 mm)] [30 by 60 inches (762 by 1524 mm)] [As indicated on Drawings] [As indicated in a schedule].

Recycled Content: [insert project requirements here %].

Required accessories: Stabilizer bars for all panels longer than 4’-11” (1500 mm)

Panel Warranty: 30-year lifetime system warranty against visible sag, mold and mildew.

Accessories.

Molding

UA-25 Aluminum molding: Channel shape, 1”x1-9/16” x1/2” (25 x 40 x 12.4).

U-2-3/32 Aluminum molding: Channel shape, 2-3/32”x7/8”x1-1/8” (53 x 22 x 28.5).

Hold down clip, for cut perimeter panels in molding

T-15 hold down clip for UA-25 molding

U-2-3/32 hold down clip for U-2-3/32 molding

CA5 Retrofit molding: For ceiling track, finished to match with ceiling panel.

CA2 Retrofit slip-on molding: For wall angle, finished to match with ceiling panel.

CA1 arrowhead reveal spacer: For cut perimeter panels into first reveal row.

CA4 round reveal spacer: For cut perimeter panels adjacent to lights or air diffuser, or for use with retrofit U-molding.

Celebration Access tool: For panel removal.

Suspension System Attachment devices.

Hanger Wire: Galvanized carbon steel; soft temper; pre-stretched; yield stress load at least three times the design load but not less than 12-gauge.

Spacing and gauge per IBC, UL and CISCA design.

Supplied and installed by ceilings subcontractor.

Disclaimer:  The USG Product Specifications contained herein are intended for use as product reference material by architects, engineers, other design professionals, contractors, building code officials, or other competent construction industry trade professionals having an interest in the selection, specification and use of products manufactured by the subsidiaries of USG Corporation. The Specifications are intended solely as technical support incident to the sale and use of USG products and not intended to be a substitute for the design review and approval of the licensed design professionals for the project.  These materials may be printed and/or transferred electronically solely as needed by the user. Because electronic text files can be modified by other parties, without notice or indication of such modifications, modification of USG Product Guide Specifications and Drawings is the sole responsibility of the Design Professional. Consult with your local USG product representative for current market values for the recycled content percentages.