Product Name: Ecophon Akusto Wall Panel™ C

Manufacturer: CertainTeed Architectural

# SECTION 09 77 23 – FABRIC-WRAPPED PANELS

## PART 1 – GENERAL

  - RELATED DOCUMENTS

- Drawings and general provisions of the contract apply to this section. This includes General and Supplementary Conditions of Division 01 (1) Specification Sections.

  - SUMMARY

- Section includes acoustic panels and suspension systems for ceilings

- Related Sections

- Section 09 20 00 (09250) - Gypsum Board, Framing & Accessories

- Division 23 (15) – Heating, Ventilating and Air Conditioning (HVAC)

- Division 26 (16) – Electrical

  - REFERENCES

- ASTM C423 – Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method

- ASTM E84 – Test Method for Surface Burning Characteristics of Building Materials

- ASTM E119 – Fire Test of Building Construction and Materials

- ASTM E795 – Practice for Mounting Test Specimens During Sound Absorption Tests

- ASTM E1264 – Classification for Acoustic Ceiling Products

- ASTM E1477 – Standard Test Method for Luminous Reflectance Factor of Acoustical Materials by Use of Integrating Sphere Reflectometer

- CAN/ULC-S102 – Method of Test for Surface Burning Characteristics of Building Materials and Assemblies

- DIN 5036 – Radiometric and Photometric Properties of Materials

- ISO 4611 – Plastic – Determination of the Effects of Exposure to Damp, Heat, Water Spray and Salt Mist

- ISO 11654 – Sound Absorbers for Use in Buildings – Rating of Sound Absorption

- ISO 14025 – Environmental Labels and Declarations -- Type III Environmental Declarations -- Principles and Procedures

- CISCA (Ceilings & Interior Systems Construction Association) – Ceilings Systems Handbook

- CISCA (Ceilings & Interior Systems Construction Association) – Acoustical Ceilings – Use and Practice

- French VOC Label - Émissions dans l'air intérieur

  - SUBMITTALS

- Product Data

- Submit manufacturer’s published technical information for each product indicated

- Shop Drawings

- Submit reflected ceiling plans drawn to scale prescribed by Architect

  - Include coordinated penetrations and ceiling-mounted items

  - Include any necessary details or drawings from the manufacturer regarding recommended installation

- Samples

- Submit representative manufacturer’s sample of each panel indicated

- Submit representative manufacturer’s sample of each suspension member indicated

- Certifications

# Provide manufacturer’s written certification that products submitted meet or exceed all specified requirements

- Provide laboratory reports that certify compliance with specified tests

- Provide third party verified life cycle information with published environmental product declaration (EPD)

  - Per ISO 14025 Environmental Labels and Declarations - Type III Environmental Declarations - Principles and Procedures

  - QUALITY ASSURANCE

- Source Limitations

- Acoustic wall element

  - Obtain each type through one source from a single manufacturer

- Suspension System

- Obtain each type through one source from a single manufacturer

- Installer Qualifications

- Must be experienced in the installation of systems similar to those specified herein

- Surface Burning Characteristics

- ASTM E1264

  - Class A

- ASTM E84 [United States]

  - Flame spread of 25 or less

  - Smoke developed of 50 or less

- CAN/ULC-S102 [Canada]

  - Flame spread of 25 or less

  - Smoke developed of 50 or less

  - DELIVERY, STORAGE AND HANDLING

- Delivery of acoustic wall products will be in the original unopened packages with the manufacturer’s label intact

- Handling and storage should be in accordance with the manufacturer’s Safety Data Sheets (SDS)

- Individual panels should be handled carefully to avoid damage

  - PROJECT CONDITIONS

- Environmental Limitations

- Install acoustic panels only in conditions that are within the manufacturer’s published limits for temperature and humidity

  - COORDINATION

- Coordinate the installation of the acoustic wall system with any and all trades whose work is impacted by that installation

- PART 2 - PRODUCTS

2.1 MANUFACTURER

- CertainTeed Architectural

- Address: 20 Moores Road, Malvern, PA 19355

- Telephone: 800-233-8990

- Web: www.certainteed.com/architectural

## 2.2 ACOUSTIC CEILING UNITS

- Acoustical Wall Panel (AWP) – [Type AWP-1]

- Name: Ecophon Wall Panel C

- Physical Characteristics

  - Core composition: high-density fiberglass

  - Size: 2700 x 600 mm [≈106-5/16 x 23-5/8 inches]

  - Thickness: 40 mm [≈1-9/16 inches]

  - Edges:

    - kerfed, wrapped

  - Finished Surface:

    - Texona [woven fiberglass fabric]

    - Super G [damage resistant woven fiberglass fabric]

  - Finished Surface Color:

    - Texona: [Acai, Cranberry, Eucalyptus, Garlic, Ginger, Goji, Licorice, Mustard, Menthol, Oyster, Peach, Pepper, Poppyseed, Sage, Scallop, Sea Sale, Tangerine, Thyme]

    - Super G: [white, grey, blue]

  - Panel Backing: fiberglass tissue

  - Recycled Content: 53%

    - 0% (pre-consumer)

    - 53% (post-consumer)

- Performance Criteria

  - Sound Absorption Rating

    - Class A absorber per ISO 11654 (40mm overall depth of system)

  - Noise Reduction Coefficient (NRC)

    - 0.95 (A-mounting) per ASTM C423

  - Sound Absorption Average (SAA)

    - 0.96 (A-mounting) per ASTM C423

  - Light Reflectance (LR) per ASTM E1477

    - Texona: Up to 0.81 (dependent upon color)

    - Super G: Up to 0.78 (dependent upon color)

  - Humidity Resistance per ISO 4611

    - Install in conditions that do not exceed 95% relative humidity and 86° F

  - Flame Spread Classification per ASTM E84, CAN/ULC-S102: Class A

- Independent Environmental Certifications

  - Environmental Product Declaration

    - Third-party verified Type III Environmental Product Declaration

      - Per ISO 14025 - Environmental Labels and Declarations - Type III Environmental Declarations -- Principles and Procedures

  - M1 emission classification

    - Building Information Foundation RTS – Emission Classification of Building Materials

  - French VOC Label

    - Émissions dans l'air intérieur

2.3 ATTACHMENT SYSTEM

- Components

  - Connect WP Profile System

    - Connect Wall Panel Profile

      - Size: 2687 mm [≈105-3/4 inches]

      - Colors: [white/textured, natural/anodized, black/textured]

    - Connect Fixing Bracket

    - Connect Direct Fixing Plate

    - Wall Panel Profile Splice

    - Connect Wall Panel Space Bar

      - Size: 2400 mm [≈94-1/2 inches]

    - Connect Wall Panel Corners: [outside corner]

      - Colors: [white/textured, grey, black/matt]

    - Connect Spline

  - Connect Thinline Profile System

    - Connect Thinline Profile

      - Size: 2678mm

      - Colors: [grey/textured, grey/textured, white/textured]

    - Connect Ambit Fixing

    - Connect Thinline Corner: [outside corner]

      - Colors: [grey/textured, grey/textured, white/textured]

    - Connect Thinline Splice

    - Connect Spline

## PART 3 – EXECUTION

3.1 EXAMINATION

- Ascertain acceptability of substrates and building conditions under which the ceiling system is to be installed. Do not proceed with the installation until any and all unacceptable conditions have been rectified.

3.3 INSTALLATION

- Install the ceiling system in accordance with the following:

- Manufacturer’s printed instructions

  - Available online at www.certainteed.com/architecturalApplicable local code requirements

- Applicable local code requirements

- Approved shop drawings

3.4 MAINTENANCE

- Replace any and all damaged ceiling system components

- Clean any and all exposed surfaces in accordance with the manufacturer’s printed instructions

# END OF SECTION

CertainTeed Architectural shall be held harmless for any damages resulting from the use of this specification guide